2.2 pricing scheme

2.2.1 Cost Estimation

Based on the DRT cost analysis method proposed by McCollom and Polin (1988), the annual total cost can be estimated using three main variables: operating hours, operating mileage, and the number of vehicles in service [1].

Annual Total Cost

= Cost per hour × Annual hours of operation

+ Cost per mile × Annual miles of operation

+ Cost per vehicle operated × Number of vehicles in service

In this study, the hourly cost is represented by driver labor cost, the mileage cost is represented by vehicle operating cost, and the fixed vehicle cost is represented by depreciation and insurance.

The German Federal Employment Agency's Entgeltatlas reports that the full-time median monthly gross salary for Busfahrer/in is 3,522 EUR [2]. Germany Trade & Invest states that the employer's social insurance contribution in Germany is approximately 21% of the employee's gross wage [3].

Therefore, c_h is set to 25 EUR/vehicle-hour. We assume that the service operates from 05:00 to 23:00, i.e., 16 operating hours per day.

For vehicle cost, S8 uses 8-seat vehicles. Therefore, the Mercedes-Benz Vito Tourer kompakt 116 CDI Pro 9G-TRONIC is selected as a representative vehicle type. According to the ADAC cost table, the total vehicle cost for this model is approximately 91.7 ct/km [4]. The fixed cost per vehicle is estimated using a depreciation cost of 646 EUR/month and an insurance cost of 195 EUR/month.

The DREAM_PACE report indicates that digital DRT systems require a travel dispatch center, in-vehicle devices, user interfaces, and communication networks. Therefore, an additional 10% is added for dispatching and back-office management costs [5].

Table 1. Cost assumptions for the S8 DRT scenario

| Parameter | Assumption | Basis |
| --- | --- | --- |
| c_h | 25 €/vehicle-hour | driver wage + employer contribution |
| c_km | 0.917 €/vehicle-km | ADAC Vito Tourer total car cost |
| c_v | 841 €/vehicle-month | depreciation + insurance |
| Overhead | × 1.10 | dispatching, customer service, administration |

Based on these assumptions, the estimated annual operating cost of the S8 scenario is 65.09 million EUR/year.

In S8, the model reports 9,995 served DRT rides per day and 5 rejected requests. Therefore, the cost-recovery fare is approximately 17.8 EUR/ride.

2.2.2 Fare Comparison

Cottbus public transport fares are 2.70 EUR for a single ticket, 5.60 EUR for a 24-hour ticket, and 39.90 EUR for a monthly ticket [6].

The daytime taxi tariff includes a base fare of 3.40 EUR, 2.60 EUR/km for the first 2 km, and 2.00 EUR/km after 2 km. Using an average passenger trip distance of 3.6 km, the estimated taxi fare is approximately 11.80 EUR [7].

To fully cover the estimated annual operating cost, the DRT fare would need to be set at approximately 18 EUR per ride.

Table 2. Fare comparison between DRT and existing transport options

| Mode / fare type | Fare level |
| --- | --- |
| Cottbus public transport single ticket | 2.70 € |
| Cottbus 24-hour ticket | 5.60 € |
| Average 3.6 km taxi trip | ≈ 11.80 € |
| Full cost-recovery DRT fare | 18 € |

This shows that the full cost-recovery fare for S8 is much higher than existing public transport fares and also higher than the estimated fare for an average 3.6 km taxi trip. Therefore, although a fare of about 18 EUR/ride could recover the full operating cost, it is not price competitive. Former bus users would likely consider it too expensive.

2.2.3 Reasons for High Cost and Cost-Reduction Measures

Driver cost accounts for about 67% of total operating cost. Passengers pay not only for their own ride, but also for the availability of the whole DRT fleet.

To reduce operating costs, a dynamic fleet operation strategy could first be adopted. More vehicles could be deployed during peak periods, while the number of active vehicles could be reduced during off-peak periods, thereby reducing driver-hours. Second, the waiting-time target could be relaxed moderately, accepting a slightly longer passenger waiting time in exchange for a lower vehicle requirement. Third, a mixed fleet could be used, with cheaper 4-seat or 6-seat vehicles assigned to low-demand areas to reduce fixed vehicle costs. In the long term, autonomous DRT could also reduce operating costs significantly by removing driver cost, but due to regulatory, safety, and technical constraints, it is unlikely to be the main solution in the short term.

References

[1] McCollom, B., & Polin, L. (1988). Cost analysis methodology for demand-responsive service.

[2] Bundesagentur fuer Arbeit. (n.d.). Busfahrer/in. Entgeltatlas. Retrieved June 22, 2026, from https://web.arbeitsagentur.de/entgeltatlas/

[3] Germany Trade & Invest. (2026). Social insurance system. Retrieved June 22, 2026, from https://www.gtai.de/en/invest/investment-guide/social-insurance-system--660212

[4] ADAC. (n.d.). Autokostenuebersicht [PDF]. Retrieved June 22, 2026, from https://assets.adac.de/Autodatenbank/Autokosten/autokostenuebersicht.pdf

[5] DREAM_PACE. (2023). D2.1.1 analysis report on DRT digital and operational innovations in CE regions and engaged areas (Final version). Interreg Central Europe.

[6] Cottbusverkehr. (n.d.). Moving Cottbus. Retrieved June 22, 2026, from https://www.cottbusverkehr.de/

[7] Taxi-Calculator.com. (n.d.). Cottbus taxi fare calculator. Retrieved June 22, 2026.